EVSW4K41

RS232 Communication and Protocol

Baud rate: 9600         Data bit: 8         Stop bit: 1          Parity bit: none

| RS232 Command |  |  |
| --- | --- | --- |
| Communication protocol: RS232 Communication Protocol |  |  |
| Baud rate: 9600 Data bit: 8 Stop bit: 1 Parity bit: none |  |  |
| 1. System Control |  |  |
| The ending mark of command is “<CR><LF>”. |  |  |
| Command | Description | Command & Feedback Example |
| #GET_FIRMWARE_VERSION | Get the firmware version. | @V1.0.0 |
| #FACTORY_RESET | Restore to factory defaults. | @FACTORY_RESET |
| #REBOOT | System reboot. | @REBOOT |
| #HELP [PARAM] | Get the command details. | #HELP SET_AV |
|  | [PARAM]=Null; Get all command list. |  |
|  | [PARAM]=Any command; Get the English description and usage of the command. | @SELECT VIDEO AND AUDIO INPUT PORT |
|  |  | #SET_AV PARAM1 |
|  |  | PARAM=A,PC1,PC2,PC3 |
|  |  | A - Airplay/Miracast |
|  |  | PC1 - HDMI1 |
|  |  | PC2 - HDMI2 |
|  |  | PC3 - TYPE-C |
| #SET_RST_WIRELESS | Reset Airplay/Miracast power | @RESET WIRELESS DEVICE |
| #SET_KEYPAD_LOCK 1 | Lock front panel buttons. | #SET_KEYPAD_LOCK 1 |
| #SET_KEYPAD_LOCK 0 | Unlock front panel buttons (Default). | #SET_KEYPAD_LOCK 0 |
| #GET_KEYPAD_LOCK | Get the locking status of the front panel buttons. | @KEYPAD_LOCK 1 |
| 2. Source Switching |  |  |
| Command | Description | Command & Feedback Example |
| #SET_AV A | Select the input source: Airplay/Miracast (Default). | @AV Airplay/Miracast |
| #SET_AV PC1 | Select the input source: PC1. | @AV PC1 |
| #SET_AV PC2 | Select the input source: PC2. | @AV PC2 |
| #SET_AV PC3 | Select the input source: PC3. | @AV PC3 |
| #GET_AV | Get the current input source. | @AV PC1 |
| #SET_AUTO_SWITCH 0 | Disable auto-switching mode. | @AUTO_SWITCH 0 |
| #SET_AUTO_SWITCH 1 | Enable auto-switching mode. | @AUTO_SWITCH 1 |
| #GET_AUTO_SWITCH | Get the auto-switching status. | @AUTO_SWITCH 1 |
| 3. CEC/RS232 Function Setting |  |  |
| The ending mark of command is “<CR><LF>”. |  |  |
| Command | Function | Command & Feedback Example |
| #SET_SYNCACT_CEC 1 | Enable the function of automatically sending CEC commands. | @SYNCACT_CEC 1 |
|  | When detecting video input signal or not detecting any video signal, the switcher will automatically send the corresponding CEC command to control the display device. |  |
| #SET_SYNCACT_CEC 0 | Disable the function of automatically sending CEC commands. | @SYNCACT_CEC 0 |
| #GET_SYNCACT_CEC | Get the function setting status of automatically sending CEC commands. | @SYNCACT_CEC 1 |
| #SET_SYNCACT_RS232 1 | Enable the function of automatically sending RS232 commands. | @SYNCACT_RS232 1 |
|  | When detecting video input signal or not detecting any video signal, the switcher will automatically send the corresponding RS232 command to control the display device. |  |
| #SET_SYNCACT_RS232 0 | Disable the function of automatically sending RS232 commands. | @SYNCACT_RS232 0 |
| #GET_SYNCACT_RS232 | Get the function setting status of automatically sending RS232 commands. | @SYNCACT_RS232 1 |
| #SET_DISPLAY 1 | Power on display device (Simultaneously sending CEC and RS232 commands to display device). | @DISPLAY 1 |
| #SET_DISPLAY 0 | Power off display device (Simultaneously sending CEC and RS232 commands to display device). | @DISPLAY 0 |
| 4. Function Setting |  |  |
| The ending mark of command is “<CR><LF>”. |  |  |
| Command | Description | Command & Feedback Example |
| #SET_OFF_CNT 1 | Set the number of sending DISPLAY OFF command to 1 time. | @OFF_CNT 1 |
| #SET_OFF_CNT 2 | Set the number of sending DISPLAY OFF command to 2 times. | @OFF_CNT 2 |
| #GET_OFF_CNT | Get the number of sending DISPLAY OFF command. | @OFF_CNT 1 |
| #SET_OFF_DELAY [PARAM] | Set the delay time of sending DISPLAY OFF command to [PARAM]. | #SET_OFF_DELAY 5 |
|  | [PARAM]=5~100 (1=100ms). | @OFF_DELAY 5 |
| #GET_OFF_DELAY | Get the delay time of sending DISPLAY OFF command. | @OFF_DELAY 5 |
| #SET_OUTPUT_HDCP [PARAM] | Set the HDCP mode of output port to [PARAM]. [PARAM]=1~3: | #SET_OUTPUT_HDCP 1 |
|  | 1 - ACTIVE |  |
|  | 2 - ON |  |
|  | 3 - OFF | @OUTPUT_HDCP 1 |
| #GET_OUTPUT_HDCP | Get the HDCP mode of output port. | @OUTPUT_HDCP 1 |
| #SET_SW_HDCP_MODE [PARAM] | Switch the input ports to support HDCP2.2 status. [PARAM]= 0/1. | #SET_SW_HDCP_MODE 1 |
|  | 0 - UNSUPPORT HDCP2.2 |  |
|  | 1 - SUPPORT HDCP2.2 | @SW_HDCP_MODE 1 |
| #GET_SW_HDCP_MODE | Get the HDCP2.2 status of input ports. | @SW_HDCP_MODE 1 |
| #UPLOAD_USER_EDID [PARAM] | Upload the user-defined EDID [PARAM]. | #UPLOAD_USER_EDID 1 |
|  | PARAM = 1 ~ 5 |  |
|  | 1 – User-defined EDID 1 |  |
|  | 2 - User-defined EDID 2 |  |
|  | 3 - User-defined EDID 3 |  |
|  | 4 - User-defined EDID 4 |  |
|  | 5 - User-defined EDID 5 |  |
|  | When the command applied, system prompts to upload the EDID file (.bin). Operation will be cancelled in 10 seconds. | @USER_EDID 1 READY |
|  |  | PLEASE SEND EDID DATA IN 10S |
|  |  | OK/ERROR |
| #SET_DTIME [PARAM1]: [PARAM2] | When not detecting video input signal, set the auto power-off time of display device to [PARAM1]: [PARAM2]. The default time is 10 minutes. | #SET_DTIME 1:30 |
|  | [PARAM1]=0~30 minutes. |  |
|  | [PARAM2]=0~1800 seconds. | @DTIME 1:30 |
| #GET_DTIME | Get the auto power-off time of display device. | @DTIME 30:0 |
| 5. Special Commands |  |  |
| Note: The below commands don’t need ending mark. |  |  |
| Command | Description | Command & Feedback Example |
| #SET_ON_[PARAM1]_ [PARAM2]:XXXX | Set the ASCII RS232 command XXXX to be sent to control the third-party device when the DISPLAY ON button is pressed. | #SET_ON_05_30:1234567 |
|  | [PARAM1] = 00~06 (Baud Rate) |  |
|  | 00 - 115200 |  |
|  | 01 - 57600 |  |
|  | 02 - 38400 |  |
|  | 03 - 19200 |  |
|  | 04 - 9600 |  |
|  | 05 - 4800 |  |
|  | 06 - 2400 |  |
|  | [PARAM2] = 00~99. The delay time of sending command. |  |
|  | XXXX: Any ASCII code (up to 48 bytes). | @BAUDRATE: 4800 |
|  |  | @DELAY TIME: 30 s |
|  |  | @DISPLAY ON TO SEND:1234567 |
| Page Break |  |  |
| Command | Description | Command & Feedback Example |
| #SET_H_ON_[PARAM1]_ [PARAM2]:XX XX | Set the HEX RS232 command XX XX to be sent to control the third-party device when the DISPLAY ON button is pressed. | #SET_H_ON_05_30:31 32 33 34 35 |
|  | [PARAM1] = 00~06 (Baud Rate) |  |
|  | 00 - 115200 |  |
|  | 01 - 57600 |  |
|  | 02 - 38400 |  |
|  | 03 - 19200 |  |
|  | 04 - 9600 |  |
|  | 05 - 4800 |  |
|  | 06 - 2400 |  |
|  | [PARAM2] = 00~99. The delay time of sending command. |  |
|  | XX XX: Any HEX code (0-9, A-F; up to 20 bytes. It must have a blank between 2 different XX). | @BAUDRATE: 4800 |
|  |  | @DELAY TIME: 30 s |
|  |  | @DISPLAY ON HEX TO SEND:31 32 33 34 35 |
| #SET_OF_[PARAM1]_ [PARAM2]:XXXX | Set the ASCII RS232 command XXXX to be sent to control the third-party device when the DISPLAY OFF button is pressed. | #SET_OF_05_30:ABCDEFG |
|  | [PARAM1] = 00~06 (Baud Rate) |  |
|  | 00 - 115200 |  |
|  | 01 - 57600 |  |
|  | 02 - 38400 |  |
|  | 03 - 19200 |  |
|  | 04 - 9600 |  |
|  | 05 - 4800 |  |
|  | 06 - 2400 |  |
|  | [PARAM2] = 00~99. The delay time of sending command. |  |
|  | XXXX: Any ASCII code (up to 48 bytes). | @BAUDRATE: 4800 |
|  |  | @DELAY TIME: 30 s |
|  |  | @DISPLAY OFF TO SEND:ABCDEFG |
| Page Break |  |  |
| Command | Description | Command & Feedback Example |
| #SET_H_OF_[PARAM1]_ [PARAM2]:XX XX | Set the HEX RS232 command XX XX to be sent to control the third-party device when the DISPLAY OFF button is pressed. | #SET_H_OF_05_30:41 42 43 44 45 |
|  | [PARAM1] = 00~06 (Baud Rate) |  |
|  | 00 - 115200 |  |
|  | 01 - 57600 |  |
|  | 02 - 38400 |  |
|  | 03 - 19200 |  |
|  | 04 - 9600 |  |
|  | 05 - 4800 |  |
|  | 06 - 2400 |  |
|  | [PARAM2] = 00~99. The delay time of sending command. |  |
|  | XX XX: Any HEX code (0-9, A-F; up to 20 bytes. It must have a blank between 2 different XX). | @BAUDRATE: 4800 |
|  |  | @DELAY TIME: 30 s |
|  |  | @DISPLAY OFF HEX TO SEND:41 42 43 44 45 |