BAMX88

8x8 Audio Processor with DSP

TCP/IP & Serial Commands

The BAMX88 supports  3rd party control. Connect a Cat5e/6 to the LAN port for TCP/IP and Telnet control, or the RS232 port of the product with serial cable,

The ASCII command list for the product are as shown below.

| ASCII Command |  |  |  |  |
| --- | --- | --- | --- | --- |
| Communication Protocol: RS-232 Baud rate: 115200 Data bit: 8 Stop bit: 1 Parity bit: none TCP/IP Port: 8000 Telnet : 23 DHCP IP Gateway Subnet Mask On 192.168.104.107 192.168.104.1 255.255.255.0 (static: 192.168.0.200 192.168.0.1 255.255.0.0 |  |  |  |  |
| x, y, z, XXX are parameters |  |  |  |  |
| Command Code | Function Description | Example | Feedback | Default Setting |
| ? | Get the list of all commands | ? | Lists all commands |  |
| help | Get the list of all commands | help | Lists all commands |  |
| get model | Get device model | get model | BAMX88 |  |
| status | Get device current status | status | Please refer to the note at the end of the list. |  |
| get version | Get firmware version | get version | MCU: V1.00.01 WEB: V1.00.01 DSP: V1.10.01 |  |
| power on | Power on the device | power on | Power on |  |
| power off | Power off the device | power off | Power:off |  |
| set system mute on/off | Set system mute on or off | set system mute on | System Mute: on |  |
| get system mute | Get system mute on or off status | get system mute | System Mute: on |  |
| get power | Get current power state | get power | power:on power:off |  |
| reboot | Reboot the device | reboot | Reboot… System Initializing... Initialization Finished! MCU: V1.00.01 WEB: V1.00.01 DSP: V1.10.01 |  |
| reset all | Reset system and network settings to default (Should type "yes" to confirm, "no" to discard) | reset all | Sure to reset system and network settings to default? Type "yes" after next prompt to confirm… |  |
| Input Setting |  |  |  |  |
| set input x lock on/off | Set input:x lock on/off x=[0-4]: 0:All inputs, 1:IN1/2, 2:IN3/4, 3:IN5/6, 4:IN7/8 | set input 1 lock on | IN1/2 lock mode: on | Off |
| get input x lock | Get input:x lock mode on/off status x=[0-4]: 0:All inputs, 1:IN1/2, 2:IN3/4, 3:IN5/6, 4:IN7/8 | get input 1 lock | IN1/2 lock mode: on |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set input x level y | Set input:x level to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-9]: 1:-35dBV, 2:-28dBV, 3:-10dBV, 4:0dBV, 5:0dBu, 6:4dBu, 7:12dBu, 8:18dBu, 9:24dBu | set input 1 level 10 | IN1 level: -35dBV | 5 |
| get input x level | Get input:x sensitivity x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 level | IN1 level: -35dBV |  |
| set input x gain y | Set input:x gain to y x=[0-10]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:IN Group A, 10:IN Group B y=[-100~12]dB Input gain value, Step=0.1dB | set input 1 gain 10 | IN1 gain: 10dB | 0 |
| get input x gain | Get input:x gain value x=[0-10]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:IN Group A, 10:IN Group B | get input 1 gain | IN1 gain: 10dB |  |
| set input x phantom power on/off | Set input:x 48V phantom power on/off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set input 3 phantom power on | IN3 phantom power: on | Off |
| get input x phantom power | Get input:x 48V phantom power status x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 3 phantom power | IN3 phantom power: on |  |
| set input x gain+ set input x gain+ y | Increase input:x gain by y x=[0-10]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:IN Group A, 10:IN Group B y=[0.1-112]:Steps, y can be empty (Step=1dB) | set input 1 gain+ set input 1 gain+ 5 | IN1 gain: 1dB IN1 gain: 5dB |  |
| set input x gainset input x gain- y | Decrease input:x gain by y x=[0-10]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:IN Group A, 10:IN Group B y=[0.1-112]:Steps, y can be empty (Step=1dB) | set input 1 gainset input 1 gain- 5 | IN1 gain: -1dB IN1 gain: -5dB |  |
| set input x invert on/off | Set input:x invert on or off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set input 1 invert on | IN1 invert on |  |
| get input x invert | Get input:x invert on/off status x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 invert | IN1 invert on |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set input x source typ y frq aa level bb | Set input:x source type y to frequency aa level bb x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-3]: 0:none, 1:sine, 2:white, 3:pink aa=[20-20000]: Frequncy value (Step=1Hz) bb=[-54~0]: Level (Step=0.1dB) | set input 1 source typ 1 frq 1000 level 0 | IN1 source type: sine frequency:1000Hz level: 0dB |  |
| get input x source | Get input:x source x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 source | IN1 source type: sine frequency:1000Hz level: 0dB |  |
| set input x mute on/off | Set input:x mute on/off x=[0-10]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:IN Group A, 10:IN Group B | set input 1 mute on | IN1 mute: on | Off |
| get input x mute | Get input:x mute on/off x=[0-10]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:IN Group A, 10:IN Group B | get input 1 mute | IN1 mute: on |  |
| set in group x member <abcdefgh> | Set input group:x member (a/b/c/d/ e/f/g/h=0-1) x=[1-2]: 1:IN Group A, 2:IN Group B a/b/c/d/e/f/g/h=0: Exclude the member a/b/c/d/e/f/g/h=1: Include the member a: IN1 b: IN2 c: IN3 d: IN4 e: IN5 f: IN6 g: IN7 h: IN8 | set in group 1 member <11110000> | IN Group A member: 11110000 | 00000000 |
| get in group x member | Get input group:x member x=[1-2]: 1:IN Group A, 2:IN Group B | get in group 1 member | IN Group A member: 11110000 |  |
| set input x exp threshold y | Set input:x expander threshold to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[-60~0]: Gain value(Step=0.1dB) | set input 1 exp threshold -30 | IN1 expander threshold: -30dB |  |
| get input x exp threshold | Get input:x expander threshold x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 exp threshold | IN1 expander threshold: -30dB |  |
| set input x exp ratio y | Set input:x expander ratio to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-20]: Ratio value(Step=0.1) | set input 1 exp ratio 2.1 | IN1 expander ratio: 2.1 |  |
| get input x exp ratio | Get input:x expander ratio x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 exp ratio | IN1 expander ratio: 2.1 |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set input x exp knee y | Set input:x expander knee to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-20]: Knee value(Step=0.1) | set input 1 exp knee 1 | IN1 expander knee: 1dB |  |
| get input x exp knee | Get input:x expander knee x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 exp knee | IN1 expander knee: 1dB |  |
| set input x exp attack y | Set input:x expander attack time to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-1000]: Attack time(Step=1ms) | set input 1 exp attack 100 | IN1 expander attack time: 100ms |  |
| get input x exp attack | Get input:x expander attack time x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 exp attack | IN1 expander attack time: 100ms |  |
| set input x exp release y | Set input:x expander release time to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-1000]: Release time (Step=1ms) | set input 1 exp release 1000 | IN1 expander release time: 1000ms |  |
| get input x exp release | Get input:x expander release time x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 exp release | IN1 expander release time: 1000ms |  |
| set input x exp active on/off | Set input:x expander active on or off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set input 1 exp active on | IN1 expander active on |  |
| get input x exp active | Get input:x expander active on/off status x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 exp active | IN1 expander active on |  |
| set input x comp threshold y | Set input:x compressor threshold to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[-60~0]: Gain value(Step=0.1dB) | set input 1 comp threshold -30 | IN1 compressor threshold: -30dB |  |
| get input x comp threshold | Get input:x compressor threshold x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 comp threshold | IN1 compressor threshold: -30dB |  |
| set input x comp ratio y | Set input:x compressor ratio to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-20]: Ratio value(Step=0.1) | set input 1 comp ratio 2.1 | IN1 compressor ratio: 2.1 |  |
| get input x comp ratio | Get input:x compressor ratio x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 comp ratio | IN1 compressor ratio: 2.1 |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set input x comp knee y | Set input:x compressor knee to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-20]: Knee value(Step=0.1) | set input 1 comp knee 1 | IN1 compressor knee: 1 |  |
| get input x comp knee | Get input:x compressor knee x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 comp knee | IN1 compressor knee: 1 |  |
| set input x comp attack y | Set input:x compressor attack time to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-1000]: Attack time(Step=1ms) | set input 1 comp attack 100 | IN1 compressor attack time: 100ms |  |
| get input x comp attack | Get input:x compressor attack time x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 comp attack | IN1 compressor attack time: 100ms |  |
| set input x comp release y | Set input:x compressor release time to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-1000]: Release time (Step=1ms) | set input 1 comp release 1000 | IN1 compressor release time: 1000ms |  |
| get input x comp release | Get input:x compressor release time x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 comp release | IN1 compressor release time: 1000ms |  |
| set input x comp active on/off | Set input:x compressor active on or off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set input 1 comp active on | IN1 compressor active on |  |
| get input x comp active | Get input:x compressor active on/ off status x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 comp active | IN1 compressor active on |  |
| set input x agc ratio y | Set input:x AGC ratio to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-20]: Ratio value(Step=0.1) | set input 1 agc ratio 2 | IN1 AGC ratio: 2 |  |
| get input x agc ratio | Get input:x AGC ratio x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc ratio | IN1 AGC ratio: 2 |  |
| set input x agc attack y | Set input:x AGC attack time to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-1000]: Attack time(Step=1ms) | set input 1 agc attack 10 | IN1 AGC attack time: 10ms |  |
| get input x agc attack | Get input:x AGC attack time x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc attack | IN1 AGC attack time: 10ms |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set input x agc attack smooth y | Set input:x AGC attack smooth to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-20]: Attack smooth (Step=0.1dB) | set input 1 agc attack smooth 0.1 | IN1 AGC attack smooth: 0.1dB |  |
| get input x agc attack smooth | Get input:x AGC attack smooth x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc attack smooth | IN1 AGC attack smooth: 0.1dB |  |
| set input x agc release y | Set input:x AGC release time to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[1-1000]: release time (Step=1ms) | set input 1 agc release 10 | IN1 AGC release time: 10ms |  |
| get input x agc release | Get input:x AGC release time x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc release | IN1 AGC release time: 10ms |  |
| set input x agc release smooth y | Set input:x AGC release smooth to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-20]: Release smooth (Step=0.1dB) | set input 1 agc release smooth 0.1 | IN1 AGC release smooth: 0.1dB |  |
| get input x agc release smooth | Get input:x AGC release smooth x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc release smooth | IN1 AGC release smooth: 0.1dB |  |
| set input x agc threshold y | Set input:x AGC threshold to y x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[-20~0]: Gain value(Step=0.1dB) | set input 1 agc threshold -30 | IN1 AGC threshold: -30dB |  |
| get input x agc threshold | Get input:x AGC threshold x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc threshold | IN1 AGC threshold: -30dB |  |
| set input x agc active on/off | Set input:x AGC active on or off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set input 1 agc active on | IN1 AGC active on |  |
| get input x agc active | Get input:x AGC active on/off status x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 agc active | IN1 AGC active on |  |
| Output Setting |  |  |  |  |
| set output x lock on/off | Set output:x lock on/off x=[0-4]: 0:All outputs, 1:OUT1/2, 2:OUT3/4, 3 OUT5/6, 4:OUT7/8 | set output 1 lock on | OUT1/2 lock mode: on | Off |
| get output x lock | Get output:x lock mode on/off status x=[0-4]: 0:All outputs, 1:OUT1/2, 2:OUT3/4, 3 OUT5/6, 4:OUT7/8 | get output 1 lock | OUT1/2 lock mode: on |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set output x level y | Set output:x level to y x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-4]: 1:20dBu, 2:4dBu, 3:0dBu, 4:-10dBV | set output 1 level 1 | OUT1 level: 20dBu | 3 |
| get output x level | Get output:x level x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 level | OUT1 level: 20dBu |  |
| set output x gain y | Set output:x gain to y x=[0-10]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8, 9:OUT Group A, 10:OUT Group B y=[-100~12]dB Output gain value, Step=0.1dB | set output 1 gain 10 | OUT1 gain: 10dB | 0 |
| get output x gain | Get output:x gain value x=[0-10]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8, 9:OUT Group A, 10:OUT Group B | get output 1 gain | OUT1 gain: 10dB |  |
| set output x gain+ set output x gain+ y | Increase output:x gain by y x=[0-10]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8, 9:OUT Group A, 10:OUT Group B y=[0.1-112]:Steps, y can be empty (Step=1dB) | set output 1 gain+ set output 1 gain+ 5 | OUT1 gain: 1dB OUT1 gain: 5dB |  |
| set output x gainset output x gain- y | Decrease output:x gain by y x=[0-10]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8, 9:OUT Group A, 10:OUT Group B y=[0.1-112]:Steps, y can be empty (Step=1dB) | set output 1 gainset output 1 gain- 5 | OUT1 gain: -1dB OUT1 gain: -5dB |  |
| set output x invert on/off | Set output:x invert on or off x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | set output 1 invert on | OUT1 invert on |  |
| get output x invert | Get output:x invert on/off status x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 invert | OUT1 invert on |  |
| set output x xovr band y frq z type a slop b | Set output:x 2-WAY crossover band y with frequency z type a slop b x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-2]: 1: High Frequency, 2:Low Frequency z=[20-20000]: Frequncy value (Step=1Hz) a=[1-3]: 1:Butterworth, 2:Bessel, 3: Linkwitz-riley b=[1-8]: 1:6dB/oct, 2:12dB/oct, 3:18dB/oct, 4:24dB/oct,5:30dB/oct, 6:36dB/oct, 7:42dB/oct, 8:48dB/oct1 (If a=3, b can only be one of the values among 2, 4, 6, 8) | set output 1 xovr band 1 frq 100 type 1 slop 1 | OUT1 2-way crossover with low frequency: 100Hz type: Butterworth slop: 6dB/oct |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| get output x xovr | Get output:x 2-WAY crossover settings x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 xovr | OUT1 2-way crossover: Low frequency:100Hz type: Butterworth slop: 6dB/oct High frequency:10000Hz type: Butterworth slop: 6dB/oct |  |
| set output x xovr band y active on/off | Set output:x 2-WAY crossover band y on or off x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-2]: 1: High Frequency, 2:Low Frequency | set output 1 xovr band 1 active on | OUT1 2-way crossover low frequency active on |  |
| get output x xovr band y active | Get output:x 2-WAY crossover band y on/off status x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-2]: 1: High Frequency, 2:Low Frequency | get output 1 xovr band 1 active | OUT1 2-way crossover low frequency active on |  |
| set output x dly y | Set output:x delay time to y x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[0-500]: Delay time (Step=1ms) | set output 1 dly 50 | OUT1 delay time: 50ms |  |
| get output x dly | Get output:x delay time x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 dly | OUT1 delay time: 50ms |  |
| set output x dly active on/off | Set output:x delay active on or off x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | set output 1 dly active on | OUT1 delay active on: 50ms |  |
| get output x dly active | Get output:x delay active on/off status x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 dly active | OUT1 delay active on |  |
| set output x mute on/off | Set output:x mute on/off x=[0-10]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8, 9:OUT Group A, 10:OUT Group B | set output 1 mute on | OUT1 mute: on | Off |
| get output x mute | Get output:x mute on/off x=[0-10]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8, 9:OUT Group A, 10:OUT Group B | get output 1 mute | OUT1 mute: on |  |
| set output x lim release y | Set output:x LIM release time to y x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-1000]: Release time (Step=1ms) | set output 1 lim release 1000 | OUT1 LIM release time: 1000ms |  |
| get output x lim release | Get output:x LIM release time x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 lim release | OUT1 LIM release time: 1000ms |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set output x lim threshold y | Set output:x LIM threshold to y x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[-60~0]: Gain value(Step=0.1dB) | set output 1 lim threshold -30 | OUT1 LIM threshold: -30dB |  |
| get output x lim threshold | Get output:x LIM threshold x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 lim threshold | OUT1 LIM threshold: -30dB |  |
| set output x lim active on/off | Set output:x LIM active on or off x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | set output 1 lim active on | OUT1 LIM active on |  |
| get output x lim active | Get output:x LIM active on/off status x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 lim active | OUT1 LIM active on |  |
| set out group x member <abcdefgh> | Set output group:x member (a/b/c/ d/e/f/g/h=0-1) x=[1-2]: 1:OUT Group A, 2:OUT Group B a/b/c/d/e/f/g/h=0: Exclude the member a/b/c/d/e/f/g/h=1: Include the member a: OUT1 b: OUT2 c: OUT3 d: OUT4 e: OUT5 f: OUT6 g: OUT7 h: OUT8 | set out group 1 member <11110000> | OUT Group A member: 11110000 | 00000000 |
| get out group x member | Get output group:x member x=[1-2]: 1:OUT Group A, 2:OUT Group B | get out group 1 member | OUT Group A member: 11110000 |  |
| Auto Mixer Setting |  |  |  |  |
| set am input x gain y | Set auto mixer input:x gain to y x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM Master y=[-100~12]dB Input gain value, Step=0.1dB | set am input 1 gain 10 | Auto mixer IN1 gain: 10dB | 0 |
| get am input x gain | Get auto mixer input:x gain value x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM Master | get am input 1 gain | Auto mixer IN1 gain: 10dB |  |
| set am input x gain+ set am input x gain+y | Increase auto mixer input:x gain by y x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM Master y=[0.1-112]:Steps, y can be empty (Step=1dB) | set am input 1 gain+ set am input 1 gain+ 5 | Auto mixer IN1 gain: 1dB Auto mixer IN1 gain: 5dB |  |
| set am input x gainset am input x gain-y | Decrease auto mixer input:x gain by y x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM Master y=[0.1-112]:Steps, y can be empty (Step=1dB) | set am input 1 gainset am input 1 gain- 5 | Auto mixer IN1 gain: -1dB Auto mixer IN1 gain: -5dB |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set am input x mute on/off | Set auto mixer input:x mute on/off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set am input 1 mute on | Auto mixer IN1 mute: on | Off |
| get am input x mute | Get auto mixer input:x mute on/off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get am input 1 mute | Auto mixer IN1 mute: on |  |
| set am input x priority y | Set auto mixer input:x priority to y x=[0-8] 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-10]:Priority level | set am input 1 priority 1 | Auto mixer IN1 priority: 1 | 5 |
| get am input x priority | Get auto mixer input:x priority value x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get am input 1 priority | Auto mixer IN1 priority: 1 |  |
| set am input x auto on/off | Set auto mixer input: x auto on/off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set am input 1 auto on | Auto mixer IN1 auto: on |  |
| get am input x auto | Get auto mixer input: x auto on/off status x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get am input 1 auto | Auto mixer IN1 auto: on |  |
| Matrix Setting |  |  |  |  |
| set output x from input y set output x from input y level z | Set output: x from input: y with level: z x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-12]: 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM, 10:AFC, 11:AEC, 12:ANS z=[-100-0]dB channel level value, Step=0.1dB | set output 1 from input 1 set output 1 from input 1 level 0 | OUT1 from IN1 OUT1 from IN1 level 0dB |  |
| set output x remove from y set output x1 x2... remove from y | Set output: x remove from input: y x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[1-12]: 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM, 10:AFC, 11:AEC, 12:ANS | set output 1 remove from 1 set output 1 2 3 4 5 remove from 1 | OUT1 remove from IN1 OUT1, OUT2 remove from IN1 |  |
| AFC Setting |  |  |  |  |
| set afc local input include x | Set AFC local input include input:x x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM | set afc local input include 1 | AFC local input include IN1 |  |
| set afc local input remove x | Set AFC local input remove input:x x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM | set afc local input remove1 | AFC local input remove IN1 |  |
| get afc local input | Get AFC local input | get afc local input | AFC local input include: IN1, IN2, AM |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| set afc band x frq y gain aa q bb | Set AFC band:x to frequency:y Gain: aa Q: bb x=[0-16]:Band number, 0: All bands y=[20-20000]: Frequncy value (Step=1Hz) aa=[-30~0]: Gain value (Step=0.1dB) bb=[0.02~16]: Q value (Step=0.01) | set afc band 1 frq 50 gain -3 q 0.7 | AFC band 1 frequency 50Hz gain -3dB Q 0.7 |  |
| get afc band x | Get AFC band:x settings x=[0-16]: Band number, 0: All bands | get afc band 1 get afc band 0 | Band Freq Gain Q Fix Bypass 1 20Hz -1.0dB 0.20 on off AFC settings: Band Freq Gain Q Fix Bypass - 20Hz -1.0dB 0.20 on off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off 10 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off - 1000Hz 0.0dB 5.00 off off |  |
| set afc fix band x on/off | Set AFC fix band on or off x=[0-16]:Band number, 0: All bands | set afc fix band 1 on | AFC fix band on |  |
| get afc fix band x | Get AFC fix band on/off status x=[0-16]: Band number, 0: All bands | get afc fix band 1 | AFC fix band on |  |
| set afc fix all on/off | Set AFC fix all on or off | set afc fix all on | AFC fix all on |  |
| get afc fix all | Get AFC fix all on/off status | get afc fix all | AFC fix all on |  |
| set afc bypass band x on/off | Set AFC bypass band on or off x=[0-16]: Band number, 0: All bands | set afc bypass band 1 on | AFC bypass band on |  |
| get afc bypass band x | Get AFC bypass band on/off status x=[0-16]: Band number, 0: All bands | get afc bypass band 1 | AFC bypass band on |  |
| set afc max depth x | Set AFC max depth to x x=[-20~0]: Gain value (Step=0.1dB) | set afc max depth -10 | AFC max depth: -10dB |  |
| get afc max depth | Get AFC max depth | get afc max depth | AFC max depth: -10dB |  |
| set afc bandwidth x | Set AFC bandwidth to x x=[1-2]: 1:Narrow, 2:Wide | set afc bandwidth 1 | AFC bandwidth: Narrow |  |
| get afc bandwidth | Get AFC bandwidth | get afc bandwidth | AFC bandwidth: Narrow |  |
| set afc reset | Reset AFC settings | set afc reset | Reset AFC settings |  |
| set afc active on/off | Set AFC acitve on or off | set afc active on | AFC active on |  |
| get afc active | Get AFC acitve on/off status | get afc active | AFC active on |  |
| AEC Setting |  |  |  |  |
| set aec local input include x | Set AEC local input include input:x x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM | set aec local input include 1 | AEC local input include IN1 |  |
| set aec local input remove x | Set AEC local input remove input:x x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM | set aec local input remove1 | AEC local input remove I N1 |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| get aec local input | Get AEC local input | get aec local input | AEC local input include: IN1, IN2, AM |  |
| set aec nlp mode x | Set AEC nonlinear processing mode to x [x=1-3]: 1:Soft, 2:Medium, 3:Aggressive | set aec nlp mode 1 | AEC nonlinear processing mode: Soft |  |
| get aec nlp mode | Get AEC nonlinear processing mode | get aec nlp mode | AEC nonlinear processing mode: Soft |  |
| set aec dly est active on/off | Set AEC delay estimation active on or off | set aec dly est active on | AEC delay estimation active on |  |
| get aec dly est active | Get AEC delay estimation active on/off status | get aec dly est active | AEC delay estimation active on |  |
| set aec ans active on/off | Set AEC ANS active on or off | set aec ans active on | AEC ANS active on |  |
| get aec ans active | Get AEC ANS active on/off status | get aec ans active | AEC ANS active on |  |
| set aec agc active on/off | Set AEC AGC active on or off | set aec agc active on | AEC AGC active on |  |
| get aec agc active | Get AEC AGC active on/off status | get aec agc active | AEC AGC active on |  |
| set aec active on/ off | Set AEC active on or off | set aec active on | AEC active on |  |
| get aec active | Get AEC active on/off status | get aec active | AEC active on |  |
| set aec remote ref include x | Set AEC remote reference include input: x x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set aec remote ref include 2 | AEC remote reference include IN2 |  |
| set aec remote ref remove x | Set AEC remote reference remove input: x x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | set aec remote ref remove 2 | AEC remote reference remove IN2 |  |
| get aec remote ref | Get AEC remote reference | get aec remote ref | AEC remote reference include IN2 |  |
| set aec remote ref level x | Set AEC remote reference level to x x=[-100~12]dB Gain value, Step=0.1dB | set aec remote ref level 0 | AEC remote reference level: 0dB |  |
| get aec remote ref level | Get AEC remote reference level | get aec remote ref level | AEC remote reference level: 0dB |  |
| ANS Setting |  |  |  |  |
| set ans local input include x | Set ANS local input include input:x x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM | set ans local input include 1 | ANS local input include IN1 |  |
| set ans local input remove x | Set ANS local input remove input:x x=[0-9]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8, 9:AM | set ans local input remove 1 | ANS local input remove IN1 |  |
| get ans local input | Get ANS local input | get ans local input | ANS local input include: IN1, IN2, AM |  |
| set ans active on/ off | Set ANS active on or off | set ans active on | ANS active on |  |
| get ans active | Get ANS active on/off status | get ans active | ANS active on |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| PEQ Setting |  |  |  |  |
| set input x eq y typ t frq z val aa q bb | Set input:x EQ index:y type t to frequency z gain aa Q bb x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-8]: EQ index 0:All t=[1-5]: 1:Parametric, 2:High Shelf, 3:Low Shelf, 4:Lowpass, 5:Highpass z=[20-20000]: Frequncy value (Step=1Hz) aa=[-15~15]: Gain value (Step=0.1dB) bb=[0.02~16]: Q value (Step=0.01) | set input 1 eq 0 typ 1 frq 200 val -10 q 0.02 | Input : 1 Band : all Type : Parametric Freq : 200Hz Gain : -10.0dB Q : 0.02 |  |
| set input x eq y active on/off | Set input:x EQ index:y active on or off x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 y=[0-8]: EQ index 0:All | set input 1 eq 1 active on | Input : 1 Band : 1 OnOff : on |  |
| get input x eq | Get input:x EQ index:y value x=[0-8]: 0:All inputs, 1:IN1, 2:IN2, 3:IN3, 4:IN4, 5:IN5, 6:IN6, 7:IN7, 8:IN8 | get input 1 eq | IN1 EQ: Band Type Freq Gain Q OnOff - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off |  |
| set output x eq y typ t frq z val aa q bb | Set output:x EQ index:y type t to frequency z gain aa Q bb x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[0-8]: EQ index 0:All t=[1-5]: 1:Parametric, 2:High Shelf, 3:Low Shelf, 4:Lowpass, 5:Highpass z=[20-20000]: Frequncy value (Step=1Hz) aa=[-15~15]: Gain value (Step=0.1dB) bb=[0.02~16]: Q value (Step=0.01) | set output 0 eq 0 typ 1 frq 200 val -15 q 0.02 | Output : 1 Band : all Type : Parametric Freq : 200Hz Gain : -15.0dB Q : 0.02 |  |
| set output x eq y active on/off | Set output:x EQ index:y active on or off x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 y=[0-8]: EQ index 0:All | set output 1 eq 1 active on | OUT1 EQ 1 active on |  |
| get output x eq | Get output:x EQ index:y value x=[0-8]: 0:All outputs, 1:OUT1, 2:OUT2, 3:OUT3, 4:OUT4, 5:OUT5, 6:OUT6, 7:OUT7, 8:OUT8 | get output 1 eq | OUT1 EQ: Band Type Freq Gain Q OnOff - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off - Parametric 32Hz 0.0dB 5.00 off |  |

| Command Code | Function Description | Example | Feedback | Default Setting |
| --- | --- | --- | --- | --- |
| Preset Setting |  |  |  |  |
| set preset x name y | Set preset (x=1~5) name to y (16 characters max) x=[1-5]: Preset 1 - Preset 5 | set preset 1 name 111 | Preset 1 name:111 |  |
| get preset x name | Get preset (x=1~5) name x=[1-5]: Preset 1 - Preset 5 | get preset 1 name | Preset 1 name:111 |  |
| set preset save x | Save the current unit’s settings to preset:x All settings except network and GPIO settings x=[1-5]: Preset 1 - Preset 5 | set preset save 1 | Save to preset 1 |  |
| set preset recall x | Recall preset:x into unit All settings except network and GPIO settings x=[1-5]: Preset 1 - Preset 5 | set preset recall 1 | Recall preset 1 |  |
| set preset clear x | Clear preset:x All settings except network and GPIO settings x=[1-5]: Preset 1 - Preset 5 | set preset clear 1 | Clear preset 1 |  |
| GPIO Setting |  |  |  |  |
| set gpio x dir y func z type aa bb cc dd ee | Set GPIO:x direction y to function z with type aa bb cc dd ee x=[0-8]: GPIO number, 0:All GPIO y=[0-2]: 0:Disable 1:Input, 2:Output z=[1-4]: Function number z=1: Channel mute z=2: System mute When y=1: Set GPIO as input aa=[1-4] aa=1: Mute on rising edge aa=2: Rising edge mute, falling edge unmute aa=3: Mute on falling edge aa=4: Falling edge mute, rising edge unmute bb=[1-2]: 1:Input, 2: Output cc=[1-8]: Channel number dd, ee should be empty When y=2: Set GPIO as output aa=[1-2]: 1:Output low, otherwise high, 2: Output high, otherwise low bb=[1-2]: 1:Input, 2: Output cc=[1-8]: Channel number dd, ee should be empty z=3: Recall preset When y=1: Set GPIO as input aa=[1-2]: 1:Rising edge, 2: Falling edge bb=[1-5]: Preset number cc, dd, ee should be empty z=4: Level setting When y=1: Set GPIO as input aa=[1-2]: 1:Rising edge, 2: Falling edge bb=[1-2]: 1:Input, 2: Output cc=[1-8]: Channel number dd=[1-2]: 1:Increment 2:Decrease ee=[1-6]: Gain step (Step=1dB) | set gpio 1 dir 2 func 1 type 1 2 1 1 1 | GPIO1: Output mode. Set to low when output channel 1 is mute, otherwise high |  |
| get gpio x | Get GPIO:x status x=[0-8]: GPIO number, 0:All GPIO | get gpio 1 | GPIO1: Output mode. Set to low when output channel 1 is mute, otherwise high |  |

| Command Code | Function Description | Example | Feedback |  | Default Setting |
| --- | --- | --- | --- | --- | --- |
| Network Setting |  |  |  |  |  |
| get ipconfig | Get the current IP configuration | get ipconfig | IP mode: DHCP IP: 192.168.62.106 Subnet mask: 255.255.255.0 Gateway: 192.168.62.1 MAC: 6C:DF:FB:0C:B3:8E (Static: 169.254.100.200 255.255.0.0 169.254.100.1) |  |  |
| get mac addr | Get network MAC address | get mac addr | MAC: 6C:DF:FB:0C:B3:8E |  |  |
| set ip mode x | Set network IP mode to static IP or DHCP x=[0-1]: 0.Static, 1.DHCP | set ip mode 0 | IP mode: Static (Please use "set net reboot" command or repower device to apply new config!) |  | 1 |
| get ip mode | Get network IP mode | get ip mode | IP mode: DHCP |  |  |
| set ip addr xxx.xxx.xxx.xxx | Set network IP address | set ip addr 192.168.1.100 | IP address: 192.168.0.100 (Please use "set net reboot" command or repower device to apply new config!) DHCP on, Device can't config static address, set DHCP off first. |  |  |
| get ip addr | Get network IP address | get ip addr | IP: 192.168.0.100 |  |  |
| set subnet xxx.xxx.xxx.xxx | Set network subnet mask | set subnet 255.255.255.0 | Subnet mask: 255.255.255.0 (Please use "set net reboot" command or repower device to apply new config!) DHCP on, Device can't config subnet mask, set DHCP off first. |  |  |
| get subnet | Get network subnet mask | get subnet | Subnet mask: 255.255.255.0 |  |  |
| set gateway xxx.xxx.xxx.xxx | Set network gateway | set gateway 192.168.1.1 | Gateway: 192.168.1.1 (Please use "set net reboot" command or repower device to apply new config!) DHCP on, Device can't config gateway, set DHCP off first. |  |  |
| get gateway | Get network gateway | get gateway | Gateway: 192.168.1.1 |  |  |
| set net reboot | Reboot network modules | set net reboot | Network reboot… Search for IP,Please wait ...! IP mode: DHCP IP: 192.168.62.106 Subnet mask: 255.255.255.0 Gateway: 192.168.62.1 TCP/IP port: 8000 Telnet port: 23 MAC: 6C:DF:FB:0C:B3:8E (Static: 169.254.100.200 255.255.0.0 169.254.100.1) |  |  |
| Command Code | Function Description | Example | Feedback | Default Setting |  |
| set net hostname xxxx | Set network hostname to xxxx (x=[32 characters max]) | set net hostname 1234 | Hostname: 1234 |  |  |
| get net hostname | Get network hostname | get net hostname | Hostname: 1234 |  |  |
| Password Setting |  |  |  |  |  |
| set admin password x | Set admin login password (x=[16 characters max]) | set admin password 123 4 | admin password: 1234 | 1234 |  |
| get admin password | Get admin login password | get admin password | admin password: 1234 |  |  |
| set user password x | Set user login password (x=[16 characters max]) | set user password 123 4 | user password: 1234 | 1234 |  |
| get user password | Get user login password | get user password | user password: 1234 |  |  |

Note: The feedback of the command of “status” is as following.

==========================================================================================

Status Info BAMX88

MCU: V1.00.01  WEB: V2.00.08  DSP: V1.10.17

Matrix     IN1     IN2     IN3     IN4     IN5     IN6     IN7     IN8     AM     AFC     AEC     ANS OUT1     0.0

OUT2                0.0

OUT3                          0.0

OUT4                                     0.0

OUT5                                                0.0

OUT6                                                          0.0

OUT7                                                                     0.0

OUT8                                                                                0.0

Inputs      Gain     Mute     Lock     Invert     Group A     Group B     Level     Phantom power

IN1          0.0        Off        Off        Off                                              0dBu     Off

IN2          0.0        Off        Off        Off                                              0dBu     Off

IN3          0.0        Off        Off        Off                                              0dBu     Off

IN4          0.0        Off        Off        Off                                              0dBu     Off

IN5          0.0        Off        Off        Off                                              0dBu     Off                           IN6          0.0        Off        Off        Off                                              0dBu     Off

IN7          0.0        Off        Off        Off                                              0dBu     Off                           IN8          0.0        Off        Off        Off                                              0dBu     Off

Outputs     Gain     Mute     Lock     Invert     Group A     Group B     Level     Delay

OUT1        0.0        Off        Off        Off                                             0dBu      0ms

OUT2        0.0        Off        Off        Off                                             0dBu      0ms

OUT3        0.0        Off        Off        Off                                             0dBu      0ms

OUT4        0.0        Off        Off        Off                                             0dBu      0ms

OUT5        0.0        Off        Off        Off                                             0dBu      0ms

OUT6        0.0        Off        Off        Off                                             0dBu      0ms

OUT7        0.0        Off        Off        Off                                             0dBu      0ms

OUT8        0.0        Off        Off        Off                                             0dBu      0ms

==========================================================================================