| GENERAL INFORMATION |  |
| --- | --- |
| SIMPLWINDOWS NAME: | Vanco 8X8 HDMI Matrix EVMX8X8 Serial |
| CATEGORY: | Matrix |
| VERSION: | 1.0 |
| SUMMARY: | This module integrates Vanco 8X8 HDMI Matrix (EVMX8X8) via RS-232. |
| GENERAL NOTES: | This module integrates the functions of a Vanco 8X8 HDMI Matrix (EVMX8X8) via RS-232. |
| CRESTRON HARDWARE REQUIRED: | 3 series processor. |
| SETUP OF CRESTRON HARDWARE: | Connect the Vanco 8X8 HDMI Matrix to a RS-232 serial port on the Crestron controller. Configure the COM Two-way serial driver to the connected RS-232 serial port with the following settings: Baud Rate - 9600 Data Bits - 8 Stop Bits - 1 Parity - N Connect "TX_SERIAL_COMMAND" to the [tx$] connection of the Two-way serial driver. Connet "RX_SERIAL_DATA" to the [rx$] connection of the Two-way serial driver. |
| VENDOR FIRMWARE: | V1.0.1 |
| VENDOR SETUP: | N/A |
| CABLE DIAGRAM: | N/A |

| CONTROL: |  |  |
| --- | --- | --- |
| POWER_ON | D | Pulse to send "Power On" command to the matrix. |
| POWER_OFF | D | Pulse to send "Power Off" command to the matrix. |
| POWER_STANDBY | D | Pulse to send "Power Standby" command to the matrix. |
| UPDATE_STATUS | D | Pulse to fetch and update the current values of the matrix's outputs. |
| SET_INPUT_D[1-8] | D | Pulse [1-8] to select an input to switch to the selected output. |
| SET_OUTPUT_D[1-9] | D | Pulse [1-9] to select an output to switch to the selected input. Pulse 9 to select all outputs. |
| SET_INPUT_A | A | Pulse [1-8] to select an input to switch to the selected output. |
| SET_OUTPUT_A | A | Pulse [1-9] to select an output to switch to the selected input. Pulse 9 to select all outputs. |
| SAVE_PRESET | A | Pulse [0-9] to save the current configuration to the pulsed number. |
| RECALL_PRESET | A | Pulse [0-9] to recall the configuration previously saved to the pulsed number. |
| CLEAR_PRESET | A | Pulse [0-9] to clear the configuration previously saved to the pulsed number. |
| RX_SERIAL_DATA | S | Connect to the [rx$] output of the Two-way serial driver. |

| FEEDBACK: |  |  |
| --- | --- | --- |
| OUTPUT1 | A | Analog feedback of the input number that output 1 is set to. |
| OUTPUT2 | A | Analog feedback of the input number that output 2 is set to. |
| OUTPUT3 | A | Analog feedback of the input number that output 3 is set to. |
| OUTPUT4 | A | Analog feedback of the input number that output 4 is set to. |
| OUTPUT5 | A | Analog feedback of the input number that output 5 is set to. |
| OUTPUT6 | A | Analog feedback of the input number that output 6 is set to. |
| OUTPUT7 | A | Analog feedback of the input number that output 7 is set to. |
| OUTPUT8 | A | Analog feedback of the input number that output 8 is set to. |
| TX_SERIAL_COMMAND | S | Connect to the [tx$] input of the Two-way serial driver. |

| TESTING: |  |
| --- | --- |
| OPS USED FOR TESTING: | RMC3 1.010.0060 |
| SIMPL WINDOWS USED FOR TESTING: | 4.03.20 |
| DEVICE DB USED FOR TESTING: | 69.05.001.00 |
| CRES DB USED FOR TESTING: | 53.05.005.00 |
| SYMBOL LIBRARY USED FOR TESTING: | 968 |
| SAMPLE PROGRAM: | Vanco EVMX8X8 and EVMX8X8 Demo |
| REVISION HISTORY: | v1.0 – Original Release |