| GENERAL INFORMATION |  |
| --- | --- |
| SIMPLWINDOWS NAME: | Vanco EVMX44SL 4X4 HDMI Matrix |
| CATEGORY: | Matrix |
| VERSION: | 1.0 |
| SUMMARY: | This module integrates Vanco 4X4 HDMI Matrix (EVMX44SL) via RS-232. |
| GENERAL NOTES: | This module integrates the functions of a Vanco 4X4 HDMI Matrix (EVMX44SL) via RS-232. |
| CRESTRON HARDWARE REQUIRED: | 3 series processor. |
| SETUP OF CRESTRON HARDWARE: | Connect the Vanco 4X4 HDMI Matrix to a RS-232 serial port on the Crestron controller. Configure the COM Two-way serial driver to the connected RS-232 serial port with the following settings: Baud Rate - 115200 Data Bits - 8 Stop Bits - 1 Parity - N Connect "TX_SERIAL_COMMAND" to the [tx$] connection of the Two-way serial driver. Connet "RX_SERIAL_DATA" to the [rx$] connection of the Two-way serial driver. |
| VENDOR FIRMWARE: | V1.0.1 |
| VENDOR SETUP: | N/A |
| CABLE DIAGRAM: | N/A |

| CONTROL: |  |  |
| --- | --- | --- |
| SET_INPUT_D[1-4] | D | Pulse [1-4] to select an input to switch to the selected output. |
| SET_OUTPUT_D[1-5] | D | Pulse [1-5] to select an output to switch to the selected input. Pulse 5 to select all outputs. |
| SET_INPUT_A | A | Pulse [1-8] to select an input to switch to the selected output. |
| SET_OUTPUT_A | A | Pulse [1-9] to select an output to switch to the selected input. Pulse 9 to select all outputs. |
| SET_SWITCH_MODE_OUTPUT[1-4] | A | Pulse [1-16] to set the output to the pulsed switch mode number: Fast switching = 1 Fade in/Fade out = 2 Split Vertical Out = 3 Split Horizontal Out = 4 Wipe Right = 5 Wipe Down = 6 Wheel Clockwise 1 = 7 Wheel Clockwise 4 = 8 Box Out = 9 Box In = 10 Circle In = 11 Circle Out = 12 Blinds Horizontal = 13 Blinds Vertical = 14 Checkerboard Across = 15 Checkerboard Down = 16 |
| SET_SWITCH_TIME_OUTPUT[1-4] | A | Pulse [1-8] to set the output to the pulsed switch time number: [0.5s] = 1 [1s] = 2 [1.5s] = 3 [2s] = 4 [2.5s] = 5 [3s] = 6 [3.5s] = 7 [4s] = 8 |
| SET_INPUT_MODE_INPUT[1-4] | A | Pulse [1-3] to set the input to the pulsed input mode number. HDMI is 1, VGA is 2, and AV is 3. |
| SET_SCALER | A | Pulse [1-4] to set the pulsed output number to scaler. |
| SET_AUTO | A | Pulse [1-4) to set the pulsed output number to AUTO. |
| SET_BEEP | A | Pulse 0 to disable beep sounds or 1 to enable beep sounds. |
| SET_BORDER | A | Pulse 0 to disable border or 1 to enable border. |
| SET_DISPLAY_MODE | A | Pulse [1-3) to set the display mode to the pulsed number: Matrix = 1 Multi-viewer = 2 Video Wall = 3 |
| SET_MULTIVIEWER_MODE | A | Pulse [1-4] to set the multi-viewer mode to the pulsed number. |
| SET_MULTIVIEWER_MAIN_INDEX[1-4] | A | Pulse [1-4] to set the multi-viewer main mode to the pulsed number. |
| SET_MULTIVIEWER_AUDIO | A | Pulse [1-4] to set the multi-viewer mode audio to the pulsed number. |
| SET_VIDEO_WALL_ADJUST_X | A | Pulse [1-32] to adjust the X value of the video wall by the pulsed number. |
| SET_VIDEO_WALL_ADJUST_Y | A | Pulse [1-32] to adjust the Y value of the video wall by the pulsed number. |
| SET_VIDEO_WALL_INPUT_CHANNEL | A | Pulse [1-4] to set the input channel of the video wall to the pulsed number. |
| RX_SERIAL_DATA | S | Connect to the [rx$] output of the Two-way serial driver. |

| FEEDBACK: |  |  |
| --- | --- | --- |
| OUTPUT1 | A | Analog feedback of the input number that output 1 is set to. |
| OUTPUT2 | A | Analog feedback of the input number that output 2 is set to. |
| OUTPUT3 | A | Analog feedback of the input number that output 3 is set to. |
| OUTPUT4 | A | Analog feedback of the input number that output 4 is set to. |
| TX_SERIAL_COMMAND | S | Connect to the [tx$] input of the Two-way serial driver. |

| TESTING: |  |
| --- | --- |
| OPS USED FOR TESTING: | RMC3 1.010.0060 |
| SIMPL WINDOWS USED FOR TESTING: | 4.03.20 |
| DEVICE DB USED FOR TESTING: | 69.05.001.00 |
| CRES DB USED FOR TESTING: | 53.05.005.00 |
| SYMBOL LIBRARY USED FOR TESTING: | 968 |
| SAMPLE PROGRAM: | Vanco EVMX44SL 4x4 HDMI Matrix |
| REVISION HISTORY: | v1.0 – Original Release |